Project title:  LoRa-based Emergency Communication Device for solo trekkers in network dead zones

Project Guide: Prof Kranti Bade

Team Members: Athish - 124A1091

Dinesh - 124A1093

Kamlesh - 124A1095

Shrivenkatesh - 124A1097

Year: Third Year

Department: Computer Engineering

Institution: SIES Graduate School of Technology

Date: 25th July 2026

Project 1: A LoRa-Based Communication System for Coordinated Response in an Earthquake

Paper Details:

- Paper Title: A LoRa-Based Communication System for Coordinated Response in an Earthquake

- First Author: Roger Pueyo Centelles

- Type: Conference Paper

- Publisher: MDPI Proceedings (UCAmI 2019)

- Year: 2019

Aim Of the Project:

- To develop a LoRaWAN-based emergency communication system for disaster situations such as earthquakes.

- To enable citizens to report their safety status to emergency response teams using predefined messages.

- To maintain communication when mobile networks and Internet services fail.

- To improve coordination between affected citizens and rescue authorities.

Equipment’s Used:

- LoRa User Nodes

- LoRa Gateways

- LoRaWAN Network Server

- Application Server

- IEEE 802.11 Wireless Mesh Network (between gateways)

- LoRa Radio Modules

- Battery-powered User Devices

- OMNeT++ Simulator

- FLoRa Simulation Framework

Working Principle:

- Citizens use portable LoRa nodes to send predefined emergency messages.

- Messages are transmitted wirelessly to nearby LoRa gateways.

- Gateways forward received packets to the network server.

- The application server processes messages and displays useful information for rescue teams.

- The complete communication system is based on the LoRaWAN star-of-stars architecture.

- Performance of the proposed system is evaluated using simulated earthquake scenarios in Coquimbo, Chile.

Major Findings:

- The proposed system successfully provides emergency communication without depending on cellular networks.

- The communication system performs efficiently for approximately 1000 user nodes.

- Increasing the number of gateways significantly improves packet delivery.

- More than one gateway receiving the same packet improves communication reliability.

- Multiple retransmissions increase successful communication but also increase packet collisions.

Flaws in the Project:

- The work is entirely simulation-based; no real hardware prototype was implemented.

- The system mainly supports predefined emergency messages and not free-text communication.

- GPS location sharing is not included.

- Direct communication between user nodes is not implemented.

- A large number of gateways are required for high communication reliability.

- Packet collisions increase rapidly when many users transmit simultaneously.

- Communication efficiency decreases significantly in dense deployments.

Full Reference:

Roger Pueyo Centelles, Felix Freitag, Roc Meseguer, Leandro Navarro, Sergio F. Ochoa, Rodrigo M. Santos, "A LoRa-Based Communication System for Coordinated Response in an Earthquake Aftermath," Proceedings of the 13th International Conference on Ubiquitous Computing and Ambient Intelligence (UCAmI 2019), MDPI Proceedings, 2019.

Project 2: Intelligent Emergency Evacuation System Based on LoRa Communication Technology and Evacuation Decision Optimization Model

Paper Details:

- Paper Title: Intelligent Emergency Evacuation System Based on LoRa Communication Technology and Evacuation Decision Optimization Model

- First Author: Not provided

- Type: Journal Article

- Publisher: Science Direct

- Year: Not provided

Aim Of the Project:

- To develop an Intelligent Emergency Evacuation System (IEES) for large public buildings.

- To improve emergency evacuation efficiency during fire accidents.

- To dynamically guide people away from hazardous areas and congested evacuation routes.

- To utilize LoRa communication technology for reliable long-range wireless data transmission.

- To optimize evacuation routes using the Evacuation Decision Optimization Model (EDOM).

- To reduce evacuation time and crowd congestion during emergencies.

Equipment’s Used:

- LoRa Communication Modules

- Multi-Sensor Fire Monitoring System (FMS)

- Central Control System (CCS)

- Wireless Evacuation Indication System (WEIS)

- Fire Sensors

- Gas Sensors

- Building Information Modeling (BIM) Platform

- Wireless Emergency Evacuation Signboards

- Visual Indicators

- Audio Warning System

- Central Monitoring Computer

Working Principle:

- Fire sensors continuously monitor the building for fire hazards.

- Sensor data is transmitted using LoRa technology to the Central Control System.

- The Central Control System analyzes the emergency situation.

- The Evacuation Decision Optimization Model calculates the safest evacuation routes.

- Updated evacuation instructions are transmitted wirelessly to evacuation signboards.

- Dynamic visual and audio guidance directs occupants away from dangerous or congested routes.

- The system continuously updates evacuation paths as the emergency develops.

Major Findings:

- The developed IEES successfully guides people away from hazardous fire locations.

- Crowd congestion at evacuation exits is significantly reduced.

- Global optimization of evacuation routes is achieved.

- LoRa communication provides reliable long-distance wireless transmission inside large buildings.

- The Wireless Evacuation Indication System operates effectively under heat radiation and dense smoke.

- Dual guidance using visual signs and sound warnings improves evacuation efficiency.

- Building structure has only a minor impact on communication response time.

- Maximum response delay observed was 14 ms, which satisfies fire safety standards.

Flaws in the Project:

- The system is designed specifically for large buildings and is not intended for outdoor environments.

- It depends on a central control system, creating a potential single point of failure.

- It is primarily focused on fire emergencies and does not address other disaster scenarios.

- No GPS-based location tracking of individuals is included.

- No direct communication between evacuees is supported.

- Installation requires extensive building infrastructure and sensor deployment.

- The proposed system may involve high installation and maintenance costs.

Full Reference:

Not provided the complete details of the publisher

Project 3: Utilization of LoRa as Support for Collaborative Systems in Mountain Tourism

Paper Details:

- Paper Title: Utilization of LoRa as Support for Collaborative Systems in Mountain Tourism

- First Author: Alejandro E. Dello Russo

- Type: Conference paper

- Publisher: IEEE ARGENCON 2020

- Year: 2020

Aim Of the Project:

- To develop a LoRa-based collaborative communication system for tourists visiting mountain parks and nature reserves.

- To assist tourists during trekking activities in areas without mobile network coverage.

- To continuously monitor the location of tourists throughout their journey.

- To facilitate search and rescue operations in case a tourist is lost or injured.

- To enable communication between tourists, park authorities, and ranger stations without relying on GPS.

- To verify the feasibility of the proposed communication system through field testing.

Equipment’s Used:

- LoRa Communication Modules

- Fixed LoRa Beacon Nodes (Faros)

- Base Station

- Tourist LoRa Communication Device

- Park Ranger LoRa Device

- Bluetooth Module

- Mobile Application

- Weather Monitoring Station

- Environmental Monitoring Sensors

- LoRa Antennas

- Prototype Transmitter

- Prototype Receiver

Working Principle:

- Tourists receive a portable LoRa communication device when entering the park.

- Fixed LoRa beacon nodes are installed along trekking routes at known locations.

- As tourists move along the trail, their devices automatically communicate with nearby beacon nodes.

- The beacon nodes forward tourist location information to the central base station.

- The base station continuously tracks tourist progress without requiring GPS.

- Tourists receive route information and emergency notifications through a mobile application connected via Bluetooth.

- During emergencies, tourists can send an SOS message.

- If no beacon is nearby, another tourist's device can relay the emergency message until it reaches a beacon and the base station.

- Rangers receive alerts and can initiate search and rescue operations based on the tourist's last recorded location.

Major Findings:

- LoRa provides reliable communication in remote mountain environments where cellular networks are unavailable.

- The beacon-based architecture enables tourist tracking without GPS.

- Multi-hop message forwarding improves the probability of emergency message delivery.

- The proposed communication protocol supports acknowledgements (ACKs) for reliable data transmission.

- Bluetooth integration allows tourists to access information through their smartphones.

- Field experiments confirmed that the proposed LoRa communication system is technically feasible.

- The system can significantly improve search and rescue efficiency in nature parks.

Flaws in the Project:

- Fixed beacon nodes must be installed throughout trekking routes, increasing deployment cost.

- The system is mainly designed for predefined trails and may not work effectively if tourists leave the marked path.

- Tourist location depends on communication with nearby beacon nodes rather than true continuous positioning.

- Communication performance decreases when beacon density is low.

- The field prototype was tested on a relatively small-scale network.

- The system requires infrastructure maintenance in remote outdoor environments.

- GPS-based real-time location sharing is not implemented.

Full Reference:

Alejandro E. Dello Russo, Julián A. Ferrari, Rodrigo M. Santos, "Utilización de LoRa como soporte de sistemas colaborativos en turismo de montaña" (Utilization of LoRa as Support for Collaborative Systems in Mountain Tourism), 2020 IEEE Congreso Bienal de Argentina (ARGENCON 2020), IEEE, 2020.

Project 4: LoRa-based Alert System for Public-safety

Paper Details:

- Paper Title: LoRa-based Alert System for Public-safety

- First Author: Subhra Shankha Bhattacherjee

- Type: Conference paper

- Publisher: URSI AP-RASC 2019

- Year: 2019

- Organization: Indian Institute of Technology (IIT) Hyderabad, India

Aim Of the Project:

- To develop a LoRa-based public safety alert system for emergency situations.

- To provide a low-cost and low-power emergency communication solution.

- To enable people to send emergency alerts without depending on smartphones.

- To notify security personnel immediately when an emergency button is pressed.

- To provide real-time navigation to the emergency location through an Android application.

- To compare the proposed system with existing commercial emergency alert devices based on power consumption and cost.

Equipment’s Used:

- Arduino Pro Mini (ATmega328P)

- Semtech SX1276 LoRa Module

- Raspberry Pi 3 B+

- Adafruit 32u4 LoRa Module

- 4G LTE Modem

- MySQL Data Logging Server

- Android Application

- Differential GPS Module

- TP4056 Lithium Battery Charging Module

- 1000 mAh Lithium-ion Battery

- 1.5 dBi Monopole Antenna

- Push Button (Emergency Switch)

- Buzzer

- Weather-proof Enclosure

- 5V Power Adapter

- Custom PCB

Working Principle:

- Emergency nodes are installed on public infrastructure such as street light poles.

- Each node contains a push button connected to a LoRa transmitter.

- When the emergency button is pressed, the Arduino wakes from sleep mode and sends an alert message to the LoRa gateway.

- The gateway receives the emergency signal and identifies the node.

- The node location is retrieved using stored GPS coordinates.

- The gateway uploads the alert to a MySQL server through a 4G LTE modem.

- The Android application immediately notifies security personnel and displays the shortest navigation route to the emergency location.

- If acknowledgement is not received within 2 seconds, the end node automatically retransmits the emergency message.

- If the server fails, the gateway sends an SMS notification as a backup.

Major Findings:

- The proposed system provides reliable long-range emergency communication using LoRa.

- The emergency device operates with very low power consumption.

- Battery life is approximately 365 days, significantly longer than commercial emergency devices.

- The end node costs only about US$15, making the system economically suitable for large-scale deployment.

- Automatic retransmission improves communication reliability.

- The Android application enables security personnel to quickly reach the emergency location.

- The proposed architecture is more power-efficient and cost-effective than existing commercial emergency alert systems.

Flaws in the Project:

- Communication is limited to a panic-button alert and does not support two-way messaging.

- The emergency node is fixed to infrastructure and is not portable.

- The paper does not evaluate the system in dense forests or network-dead zones.

Full Reference:

Subhra Shankha Bhattacherjee, Sanju Kumar NT, Banda Santhosh Reddy, P. Rajalakshmi, "LoRa-based Alert System for Public-safety," URSI Asia-Pacific Radio Science Conference (AP-RASC 2019), New Delhi, India, March 9–15, 2019.

Project 5: Enhanced Soldier Support System (ES3) Utilizing LoRa Technology with Emergency SOS Feature

Paper Details:

- Paper Title: Enhanced Soldier Support System (ES3) Utilizing LoRa Technology with Emergency SOS Feature

- First Author: Rajan Singh

- Type: Conference paper

- Publisher: IEEE International Conference on Communication, Computing and Energy Efficient Technologies (I3CEET)

- Year: 2024

- Organization: Rajan Singh, Sandeep Kumar Singh, Pulkit Singh, Kiran Kumar Bhadavath, M. Raju Naik, Shrikant Upadhyay

Aim Of the Project:

- To develop an Enhanced Soldier Support System (ES3) using LoRa technology.

- To monitor soldiers' real-time health parameters and geographical location during military operations.

- To provide an emergency SOS feature for immediate assistance.

- To improve battlefield communication in remote and mountainous regions where conventional communication is unreliable.

- To enhance soldier safety through continuous health monitoring and real-time location tracking

Equipment’s Used:

- Arduino ATmega328P Microcontroller

- LoRa Transmitter and Receiver Modules

- LoRa Gateway

- GPS Module (NEO-6M)

- MAX30100 SpO₂ and Heart Rate Sensor

- DHT11 Temperature and Humidity Sensor

- MEMS Accelerometer

- SOS Push Button

- Buzzer

- LCD Display

- Power Supply (12V, 1A)

- Computer/Laptop (Monitoring Station)

Working Principle:

- Health parameters such as heart rate, SpO₂, temperature, and humidity are continuously monitored using wearable sensors.

- The GPS module continuously tracks the soldier's real-time location.

- The Arduino microcontroller processes all sensor readings.

- The processed data is transmitted to the base station using LoRa communication.

- The base station displays the soldier's health status and GPS location on a monitoring system.

- In emergency situations, the soldier presses the SOS button.

- The SOS alert, along with the soldier's health data and GPS coordinates, is immediately transmitted to the control room.

- The system updates health and location information approximately every second.

Major Findings:

- LoRa provides reliable long-range communication suitable for military operations.

- Real-time monitoring of soldiers' health improves emergency response capability.

- GPS accurately provides soldier location even in remote and mountainous regions.

- The SOS feature enables immediate emergency notification to the control room.

- Continuous monitoring helps identify injuries or abnormal health conditions early.

- The system provides a lightweight and cost-effective alternative to complex monitoring systems.

- The proposed system improves soldier safety without relying on conventional communication infrastructure.

Flaws in the Project:

- Communication is limited to soldier-to-base station architecture.

- No LoRa mesh networking is implemented for extending communication range.

- The system depends on a centralized monitoring station.

- No communication between individual soldiers is supported.

- Encryption and communication security mechanisms are not described.

Full Reference:

Rajan Singh, Sandeep Kumar Singh, Pulkit Singh, Kiran Kumar Bhadavath, M. Raju Naik, Shrikant Upadhyay, "Enhanced Soldier Support System (ES3) Utilizing LoRa Technology with Emergency SOS Feature," 2024 International Conference on Communication, Computing and Energy Efficient Technologies (I3CEET), IEEE, 2024.